Project 1:Spartan6 - DSP48A1

NAME: kerolos Nasser Adly

# 1:RTL :

## 1-DSP.V

module DSP(A,B,C,D,

clk,CARRYIN,

OPMODE,BCIN,RSTA,RSTB,RSTC,RSTD,RSTP,RSTM,RSTCARRYIN,RSTOPMODE,

CEA,CEB,CEC,CED,CEP,CEM,CECARRYIN,CEOPMODE,

PCIN,BCOUT,POUT,

P,M,CARRYOUT,CARRYOUTF);

/*

parameters for DSP48A1

*/

parameter A0REG=0;

parameter A1REG=1;

parameter B0REG=0;

parameter B1REG=1;

parameter CREG=1;

parameter DREG=1;

parameter MREG=1;

parameter PREG=1;

parameter CARRYINREG=1;

parameter CARRYOUTREG=1;

parameter OPMODEREG=1;

parameter CARRYINSEL="OPMODE[5]";//or "CARRYIN"

parameter B_INPUT="DIRECT";//or "CASCADE"

parameter RSTTYPE="SYNC";//or "ASYNC"

/*

Inputs for DSP48A1

*/

input [17:0] A;

input [17:0] B;

input [17:0] BCIN;

input [47:0] C;

input [17:0] D;

input CARRYIN;

input clk;

input [7:0] OPMODE;

input CEA,CEB,CEC,CED,CEP,CEM,CECARRYIN,CEOPMODE;

input RSTA,RSTB,RSTC,RSTD,RSTP,RSTM,RSTCARRYIN,RSTOPMODE;

input [47:0] PCIN;

/*

Outputs for DSP48A1

*/

output  [35:0] M;

output [47:0] P;

output CARRYOUT;

output CARRYOUTF;

output [47:0] POUT;

output [17:0] BCOUT;

//PART1: REGs for A,B,C,D,OPMODE

//Opmode reg

wire [7:0] OPMODE_reg;

REG_pipe #(.width(8),.REG_EN(OPMODEREG),.REG_type(RSTTYPE)) OPMODE_REG(.D(OPMODE),.Q(OPMODE_reg),.rst(RSTOPMODE),.clk(clk),.CE(CEOPMODE));

//B input mux

wire [17:0] B_Cascade;

generate

if(B_INPUT=="DIRECT") begin

assign B_Cascade = B;

end

else if(B_INPUT=="CASCADE") begin

assign B_Cascade = BCIN;

end

else begin

assign B_Cascade = 18'b0;

end

endgenerate

//Regs for A,B,C,D

wire [17:0] A0_reg,B0_reg,D0_reg;

wire [47:0] C0_reg;

REG_pipe #(.width(18),.REG_EN(A0REG),.REG_type(RSTTYPE)) A0_REG(.D(A),.Q(A0_reg),.rst(RSTA),.clk(clk),.CE(CEA));

REG_pipe #(.width(18),.REG_EN(B0REG),.REG_type(RSTTYPE)) B0_REG(.D(B_Cascade),.Q(B0_reg),.rst(RSTB),.clk(clk),.CE(CEB));

REG_pipe #(.width(18),.REG_EN(DREG),.REG_type(RSTTYPE)) D0_REG(.D(D),.Q(D0_reg),.rst(RSTD),.clk(clk),.CE(CED));

REG_pipe #(.width(48),.REG_EN(CREG),.REG_type(RSTTYPE)) C0_REG(.D(C),.Q(C0_reg),.rst(RSTC),.clk(clk),.CE(CEC));

//Pre_ADDER/SUBTRACTOR

wire [17:0] pre_adder_out;

assign pre_adder_out = OPMODE_reg[6] ? (D0_reg - B0_reg) : (D0_reg + B0_reg);

wire [17:0] pre_adder_out_mux;

assign pre_adder_out_mux = OPMODE_reg[4] ? pre_adder_out:B0_reg;

//PART2: MULTIPLIER

wire [17:0] B1_reg;

wire [17:0] A1_reg;

REG_pipe #(.width(18),.REG_EN(B1REG),.REG_type(RSTTYPE)) B1_REG(.D(pre_adder_out_mux),.Q(B1_reg),.rst(RSTB),.clk(clk),.CE(CEB));

REG_pipe #(.width(18),.REG_EN(A1REG),.REG_type(RSTTYPE)) A1_REG(.D(A0_reg),.Q(A1_reg),.rst(RSTA),.clk(clk),.CE(CEA));

assign BCOUT = B1_reg;

wire [35:0] M_reg;

wire [35:0] M_out;

assign M_reg = A1_reg * B1_reg;

REG_pipe #(.width(36),.REG_EN(MREG),.REG_type(RSTTYPE)) M_REG(.D(M_reg),.Q(M_out),.rst(RSTM),.clk(clk),.CE(CEM));

assign M = M_out;

//PART3: POST_ADDER/SUBTRACTOR

//D:A:B Concatenation

wire [47:0] DAB_concat;

assign DAB_concat = {D0_reg[11:0],A1_reg[17:0],B1_reg[17:0]};

//mux X

reg [47:0] mux_X_out;

always@(*) begin

case(OPMODE_reg[1:0])

2'b00: mux_X_out = 48'b0;

2'b01: mux_X_out = {12'b0,M};

2'b10: mux_X_out = P;

2'b11: mux_X_out = DAB_concat;

default: mux_X_out = 48'b0;

endcase

end

//mux Z

reg [47:0] mux_Z_out;

always@(*) begin

case(OPMODE_reg[3:2])

2'b00: mux_Z_out = 48'b0;

2'b01: mux_Z_out = PCIN;

2'b10: mux_Z_out = P;

2'b11: mux_Z_out = C0_reg;

default: mux_Z_out = 48'b0;

endcase

end

//Carry in for post adder/subtractor

wire post_adder_carry_in_mux;

generate

if(CARRYINSEL=="OPMODE[5]") begin

assign post_adder_carry_in_mux = OPMODE_reg[5];

end

else if(CARRYINSEL=="CARRYIN") begin

assign post_adder_carry_in_mux = CARRYIN;

end

endgenerate

wire CIN_reg;

REG_pipe #(.width(1),.REG_EN(CARRYINREG),.REG_type(RSTTYPE)) POST_ADDER_CARRY_IN_REG(.D(post_adder_carry_in_mux),.Q(CIN_reg),.rst(RSTCARRYIN),.clk(clk),.CE(CECARRYIN));

//POST adder/subtractor

wire [47:0] post_adder_out;

wire carry_out_from_post_adder;

assign {carry_out_from_post_adder,post_adder_out} = OPMODE_reg[7] ? (mux_Z_out - (mux_X_out + CIN_reg)) : (mux_Z_out + (mux_X_out + CIN_reg));

//Carry out reg

wire carry_out_reg;

REG_pipe #(.width(1),.REG_EN(CARRYOUTREG),.REG_type(RSTTYPE)) CARRYOUT_REG(.D(carry_out_from_post_adder),.Q(carry_out_reg),.rst(RSTCARRYIN),.clk(clk),.CE(CECARRYIN));

assign CARRYOUT = carry_out_reg;

assign CARRYOUTF = carry_out_from_post_adder;

//P reg

wire [47:0] P_reg;

REG_pipe #(.width(48),.REG_EN(PREG),.REG_type(RSTTYPE)) P_REG(.D(post_adder_out),.Q(P_reg),.rst(RSTP),.clk(clk),.CE(CEP));

assign P = P_reg;

assign POUT = P_reg;

endmodule

## 2-REG_pipe.v

module REG_pipe(D,Q,rst,clk,CE);

parameter width=18;

parameter REG_EN=1;

parameter REG_type="SYNC";

input [width-1:0] D;

output reg [width-1:0] Q;

input rst;

input clk;

input CE;

generate

if(REG_EN) begin

if(REG_type=="SYNC") begin

always@(posedge clk) begin

if (rst) begin

Q <= 0;

end else if (CE) begin

Q <= D;

end

end

end

else if(REG_type=="ASYNC") begin

always@(posedge clk or posedge rst) begin

if (rst) begin

Q <= 0;

end else if (CE) begin

Q <= D;

end

end

end

end

else  begin

//no reg

always@(*) begin

Q = D;

end

end

endgenerate

endmodule

# 2-TEST BENCH

module DSP_tb;

// signal declaration

reg [17:0] A,B,D;

reg [17:0] BCIN;

reg [47:0] C,PCIN;

reg [7:0] OPMODE;

reg CARRYIN;

reg clk;

reg CEA,CEB,CEC,CED,CEP,CEM,CECARRYIN,CEOPMODE;

reg RSTA,RSTB,RSTC,RSTD,RSTP,RSTM,RSTCARRYIN,RSTOPMODE;

wire  [35:0] M;

wire [47:0] P;

wire CARRYOUT,CARRYOUTF;

wire [47:0] POUT;

wire [17:0] BCOUT;

// Instantiate the DSP module

DSP DUT(.A(A),.B(B),.C(C),.D(D),

.clk(clk),.CARRYIN(CARRYIN),

.OPMODE(OPMODE),.BCIN(BCIN),.RSTA(RSTA),.RSTB(RSTB),.RSTC(RSTC),.RSTD(RSTD),.RSTP(RSTP),.RSTM(RSTM),.RSTCARRYIN(RSTCARRYIN),.RSTOPMODE(RSTOPMODE),

.CEA(CEA),.CEB(CEB),.CEC(CEC),.CED(CED),.CEP(CEP),.CEM(CEM),.CECARRYIN(CECARRYIN),.CEOPMODE(CEOPMODE),

.PCIN(PCIN),

.BCOUT(BCOUT),

.POUT(POUT),

.P(P),

.M(M),

.CARRYOUT(CARRYOUT),

.CARRYOUTF(CARRYOUTF)

);

// Clock generation

initial begin

clk = 0;

forever #5 clk = ~clk; // 100MHz clock

end

// Test stimulus

initial begin

//reset operations

RSTA = 1; RSTB = 1; RSTC = 1; RSTD = 1; RSTP = 1; RSTM = 1; RSTCARRYIN = 1; RSTOPMODE = 1;

CEA = $random;

CEB = $random;

CEC = $random;

CED = $random;

CEP = $random;

CEM = $random;

CECARRYIN = $random;

CEOPMODE = $random;

A=$random;

B=$random;

C=$random;

D=$random;

BCIN=$random;

OPMODE=$random;

CARRYIN=$random;

PCIN=$random;

// give reset time for pipeline

repeat(3) @(negedge clk);

if(M !== 36'b0 || P !== 48'b0 || CARRYOUT !== 1'b0 || CARRYOUTF !== 1'b0 || POUT !== 48'b0 || BCOUT !== 18'b0) begin

$display("Reset failed!");

end else begin

$display("Reset successful.");

end

// Release reset

RSTA = 0; RSTB = 0; RSTC = 0; RSTD = 0; RSTP = 0; RSTM = 0; RSTCARRYIN = 0; RSTOPMODE = 0;

CEA = 1;

CEB = 1;

CEC = 1;

CED = 1;

CEP = 1;

CEM = 1;

CECARRYIN = 1;

CEOPMODE = 1;

//path 1:

A=20; B=10; C=350; D=25;

OPMODE=8'b11011101;

CARRYIN=$random; BCIN=$random; PCIN=$random;

repeat(4) @(negedge clk);

if(P !== 48'h32 || POUT !== 48'h32  || BCOUT !== 18'hf || M !== 36'h12c || CARRYOUTF !== 1'b0 || CARRYOUT !== 1'b0) begin

$display("Test case 1 failed!");

$display("Expected P=32, M=12c, BCOUT=f, CARRYOUT=0, CARRYOUTF=0");

$display("Got P=%h, M=%h, BCOUT=%h, CARRYOUT=%b, CARRYOUTF=%b", P, M, BCOUT, CARRYOUT, CARRYOUTF);

$stop;

end else begin

$display("Test case 1 passed.");

end

//path 2:

A=20; B=10; C=350; D=25;

OPMODE=8'b00010000;

CARRYIN=$random; BCIN=$random; PCIN=$random;

repeat(3) @(negedge clk);

if(P !== 48'h0 || POUT !== 48'h0  || BCOUT !== 18'h23 || M !== 36'h2bc || CARRYOUTF !== 1'b0 || CARRYOUT !== 1'b0) begin

$display("Test case 2 failed!");

$display("Expected P=0, M=2bc, BCOUT=23, CARRYOUT=0, CARRYOUTF=0");

$display("Got P=%h, M=%h, BCOUT=%h, CARRYOUT=%b, CARRYOUTF=%b", P, M, BCOUT, CARRYOUT, CARRYOUTF);

$stop;

end else begin

$display("Test case 2 passed.");

end

//path 3:

A=20; B=10; C=350; D=25;

OPMODE=8'b00001010;

CARRYIN=$random; BCIN=$random; PCIN=$random;

repeat(3) @(negedge clk);

if(P !== 48'h0 || POUT !== 48'h0  || BCOUT !== 18'ha || M !== 36'hc8 || CARRYOUTF !== 1'b0 || CARRYOUT !== 1'b0) begin

$display("Test case 3 failed!");

$display("Expected P=0, M=c8, BCOUT=a, CARRYOUT=0, CARRYOUTF=0");

$display("Got P=%h, M=%h, BCOUT=%h, CARRYOUT=%b, CARRYOUTF=%b", P, M, BCOUT, CARRYOUT, CARRYOUTF);

$stop;

end else begin

$display("Test case 3 passed.");

end

//path 4:

A=5; B=6; C=350; D=25;

OPMODE = 8'b10100111;

CARRYIN=$random; BCIN=$random; PCIN=3000;

repeat(3) @(negedge clk);

if(P !== 48'hfe6fffec0bb1 || POUT !== 48'hfe6fffec0bb1  || BCOUT !== 18'h6 || M !== 36'h1e || CARRYOUTF !== 1'b1 || CARRYOUT !== 1'b1) begin

$display("Test case 4 failed!");

$display("Expected P=fe6fffec0bb1, M=1e, BCOUT=6, CARRYOUT=1, CARRYOUTF=1");

$display("Got P=%h, M=%h, BCOUT=%h, CARRYOUT=%b, CARRYOUTF=%b", P, M, BCOUT, CARRYOUT, CARRYOUTF);

$stop;

end else begin

$display("Test case 4 passed.");

end

$display("All test cases passed.");

$stop;

end

endmodule

# 3 DO.File

# 4 QuestaSim Snippets:

## Path 1:

## Path2

## Path 3 & Path 4

# 5 Constraint File:

# 6 Elaboration (“Messages” tab & Schematic snippets)

# 7-Synthesis (“Messages” tab, Utilization report, timing report & Schematic snippets)

# 8_ Implementation (“Messages” tab, Utilization report, timing report & device snippets)

# 9_Linting (snippets showing no errors)